# KISSsoft System Module 2025

### User friendly KISSsoft look and feel Multi windows operations

In KISSsoft® System Module windows and tabs may be arranged over several screens to accelerate the work flow and provide visibility of the system topology while optimizing individual components.

### 3D view Automatically created from part properties Allows for import of CAD data for collision check

Whatever topology is defined in the sketcher or tree structure of the model and whatever gear and shaft details are designed, the 3D viewer allows for a fast check of the arrangement and use of design space.

### Exciting frequences Gear mesh, hunting and assembly frequencies Bearing fundamental and passing frequencies

Knowing gear and bearing frequencies helps for fault detection to prevent failure or plan maintenance actions. Frequencies are calculated over a speed range, considering harmonics and sidebands.

### Topology definition Definition as a schematic in the sketcher Use mouse and keyboard to model

The sketcher allows for definition of the topology of a gearbox or transmission as if using pen and paper. Using mouse and keyboard, the system is drawn on a grid.

### Annotations Visibility of key component properties Element labels showing element names

Annotations give a quick overview of key parts data like number of teeth and module or shaft total length  or bearing designation This simplifies communication in design reviews or when explaining the design.

### Tabular data summaries List of component properties On subsystem or system level

Key data per component type is summarized in tables. The tables inform also about the association of a part, e.g. for a bearing on which shaft it is located. In future, tables may be exported, and the content changed.

### Forced response analysis Excitation from gear mesh and other Resulting bearing forces over time

The forced response analysis considers excitation from the transmission error in all gear meshes, shaft imbalance and torque ripple. Time dependent bearing forces allow for calculation of housing noise.

### System deformation 3D display of deformed components For assessment of gear mesh misalignment

The system deformation is visualized. This helps to understand key contributions to gear misalignment, be it bearing, shaft, gear body or housing deflection.

A test version is available upon request from our website at: www.kisssoft.com/trial